**********************************************************************

DESIGNED AT	  : EINFOCHIPS LIMITED

CUSTOMER	  : EINFOCHIPS LIMITED

PCB REF.	  : C6457_LCEVM REV:02

JOB REF	  : EINF4219

DATE          : 14.10.2010

**********************************************************************

FILE DETAILS

**********************************************************************

Gerber format: 2.5, Leading zeroes suppressed, Absolute, English, RS274X

Drill format: 2.5, Trailing zeroes suppressed, Absolute, English

GERBER FILE : 4219_C6457_LCEVM_DFM_02_20101014.ZIP (ZIP FILE CONTAINING ALL GERBER FILES AND NC DRILL DATA)

GERBER FILES:

-------------

L1.GBR		- LAYER 1 PRIMARY SIDE

L2P.GBR		- LAYER 2 GND LAYER NEGATIVE

L3S.GBR		- LAYER 3 SIGNAL LAYER POSITIVE

L4P.GBR		- LAYER 4 POWER LAYER NEGATIVE

L5S.GBR		- LAYER 5 SIGNAL LAYER POSITIVE

L6P.GBR 		- LAYER 6 POWER LAYER NEGATIVE

L7P.GBR 		- LAYER 7 GND LAYER NEGATIVE

L8S.GBR		- LAYER 8 SIGNAL LAYER POSITIVE

L9P.GBR 		- LAYER 9 POWER LAYER NEGATIVE

L10S.GBR		- LAYER 10 SIGNAL LAYER POSITIVE

L11P.GBR 		- LAYER 11 GND LAYER NEGATIVE

L12.GBR		- LAYER 12 SECONDARY SIDE

SMPR.GBR		- SOLDER MASK PRIMARY

SMSE.GBR		- SOLDER MASK SECONDARY

SLPR.GBR		- SILKSCREEN PRIMARY

SLSE.GBR		- SILKSCREEN SECONDARY

DRILL FILE:

-----------

PTH.DRL		- NC DRILL DATA PTH

NPTH.DRL		- NC DRILL DATA NPTH

PTH_SLOTS.ROU	- MILL DATA PTH_SLOT

MANUFACTURING DETAILS OF C6457_LCEVM REV:02

1) NO. OF LAYERS           : 12L

2) BOARD SIZE              : 180.6 x 73.5 MM

3) FINAL BOARD THICKNESS   : AS PER FAB DRAWING

4) MINIMUM DRILL SIZE      : 8 MIL

5) MAXIMUM DRILL SIZE      : 138 MIL

6) MINIMUM TRACK THICKNESS : 4 MIL

7) MINIMUM TRACK CLEARANCE : 4.40 MIL

8) COPPER THICKNESS        : AS PER FAB DRAWING

9) FINISH                  : AS PER FAB DRAWING

10)BOARD MATERIAL		   : AS PER FAB DRAWING

OTHER FILES:

------------

FAB.GBR	      - FAB DRAWING

Rout.GBR	      - OUTLINE

Ipc netlist.IPC   - IPC NETLIST D356

4219_C6457_LCEVM_odb++.tgz	- ODB++ DATA

NOTE:

1)THIS IS A CONTROLLED IMPEDANCE BOARD. FOR MORE DEAILS PLEASE REFER FAB DRAWING.

2)MASK PADS HAS BEEN INCREASED BY 3MIL WITH RESPECT TO DATABASE AT CAM LEVEL AS PER REQUEST

3)Einfochips and TI logo has been added at Gerber level for more clarity.

4)SOLDER MASK OPENING FOR U7 as per shape deleted at Gerber level.

For any queries on this design please contact

Design Engineer :Suganya

Email id :suganya@siennaecad.com

Mobile NO :

Tel No : 30410760

CAM Engineer :ramkumar.g\Umesh.g

Email id : ramkumar.g@siennaecad.com\umesh.g@siennaecad.com

Mobile No:

Tel N0: 30410755