**********************************************************************

CUSTOMER	  : EINFOCHIPS

PCB REF	  : K2E EVM CARD  Rev:02

JOB REF	  : EINF4982B

DATE          : 04.03.2014

**********************************************************************

FILE DETAILS

**********************************************************************

Gerber format: 2.5, Leading zeroes suppressed, Absolute, English, RS274X

Drill format: 2.5, Trailing zeroes suppressed, Absolute, English

GERBER FILE : EINF4982B_K2E_EVM_CARD_DFM_20140304.ZIP

(ZIP FILE CONTAINING ALL GERBER FILES AND NC DRILL DATA)

GERBER FILES:

-------------

L1.GBR		-    LAYER 1 PRIMARY SIDE

L2P.GBR		-    LAYER 2 GROUND PLANE POSITIVE

L3S.GBR		-    LAYER 3 SIGNAL LAYER POSITIVE

L4P.GBR 		-  LAYER 4 POWER PLANE POSITIVE

L5S.GBR		-  LAYER 5 SIGNAL LAYER POSITIVE

L6P.GBR 		-  LAYER 6 GROUND PLANE POSITIVE

L7P.GBR 		-  LAYER 7 GROUND PLANE POSITIVE

L8S.GBR		-  LAYER 8 SIGNAL LAYER POSITIVE

L9P.GBR 		-  LAYER 9 POWER PLANE POSITIVE

L10S.GBR		-  LAYER 10 SIGNAL LAYER POSITIVE

L11P.GBR 		-  LAYER 11 GROUND PLANE POSITIVE

L12.GBR		-    LAYER 12 SECONDARY SIDE

SMPR.GBR		-    SOLDER MASK PRIMARY

SMSE.GBR		-    SOLDER MASK SECONDARY

SLPR.GBR		-    SILKSCREEN PRIMARY

SLSE.GBR		-    SILKSCREEN SECONDARY

DRILL FILE:

-----------

PTH.DRL		- NC DRILL DATA PTH

PTH_SLOTS.GBR	- NC DATA PTH_SLOTS

NPTH.DRL		- NC DRILL DATA NPTH

NPTH_SLOTS.GBR	- NC DATA NPTH_SLOTS

MANUFACTURING DETAILS OF K2E EVM CARD  Rev:02

1) NO. OF LAYERS           : 12L

2) BOARD SIZE              : 7.1 X 5.8 INCH

3) FINAL BOARD THICKNESS   : 63.036 MILS

4) MINIMUM DRILL SIZE      : 8 MIL

5) MAXIMUM DRILL SIZE      : 156 MIL

6) MINIMUM TRACK THICKNESS : 4.00 MIL

7) MINIMUM TRACK CLEARANCE : 3.80 MIL

8) COPPER THICKNESS        : AS PER FAB DRAWING

9) FINISH                  : AS PER FAB DRAWING

10)Material                : AS PER FAB DRAWING

OTHER LAYERS

------------

FAB.GBR		- FAB DRAWING

ROUT.GBR		- OUTLINE

IPC NETLIST.IPC 	- IPC D 356 NETLIST

K2E Stackup_20131011.xls    - Stackup details

EINF4982B_K2E_EVM_CARD_odb++.tgz	    - ODB++ DATA

NOTE:

1)The following nets are shorted intentionally

$Net16            AGND,GND

2)THIS IS A CONTROLLED IMPEDANCE BOARD.FOR DETAILS PLEASE REFER FAB DRAWING.

3) MASK PADS HAS BEEN INCREASED BY 3MIL WITH RESPECT TO DATABASE AT CAM LEVEL AS PER

REQUEST.

4) MASK PADS HAS BEEN INCREASED 2 MILS FOR U48.

5)PRESS FIT CONNECTORS ARE USED  IN THE BOARD.