**********************************************************************

DESIGNED AT	  : SIENNAECAD TECHNOLOGIES,PVT LTD.,

BANGALORE - 560 085.

CUSTOMER	  : EINFOCHIPS

PCB REF	  : K2L EVM CARD  REV:02

JOB REF	  : EINF4984B

DATE          : 18.06.2014

**********************************************************************

FILE DETAILS

**********************************************************************

Gerber format: 2.5, Leading zeroes suppressed, Absolute, English, RS274X

Drill format: 2.5, Trailing zeroes suppressed, Absolute, English

GERBER FILE : EINF4984B_K2L_EVM_CARD_DFM_02_20140618.ZIP

(ZIP FILE CONTAINING ALL GERBER FILES AND NC DRILL DATA)

GERBER FILES:

-------------

L1.GBR		-    LAYER 1 PRIMARY SIDE

L2P.GBR		-    LAYER 2 GROUND PLANE POSITIVE

L3S.GBR		-    LAYER 3 SIGNAL LAYER POSITIVE

L4P.GBR 		-  LAYER 4 POWER PLANE POSITIVE

L5S.GBR		-  LAYER 5 SIGNAL LAYER POSITIVE

L6P.GBR 		-  LAYER 6 GROUND PLANE POSITIVE

L7P.GBR 		-  LAYER 7 GROUND PLANE POSITIVE

L8S.GBR		-  LAYER 8 SIGNAL LAYER POSITIVE

L9P.GBR 		-  LAYER 9 POWER PLANE POSITIVE

L10S.GBR		-  LAYER 10 SIGNAL LAYER POSITIVE

L11P.GBR 		-  LAYER 11 GROUND PLANE POSITIVE

L12.GBR		-    LAYER 12 SECONDARY SIDE

SMPR.GBR		-    SOLDER MASK PRIMARY

SMSE.GBR		-    SOLDER MASK SECONDARY

SLPR.GBR		-    SILKSCREEN PRIMARY

SLSE.GBR		-    SILKSCREEN SECONDARY

DRILL FILE:

-----------

PTH.DRL		- NC DRILL DATA PTH & SLOTS

NPTH.DRL		- NC DRILL DATA NPTH & SLOTS

MANUFACTURING DETAILS OF K2L EVM CARD  REV:02

1) NO. OF LAYERS           : 12L

2) BOARD SIZE              : 180.6 X 148.5 MM

3) FINAL BOARD THICKNESS   : 67.036 MIL

4) MINIMUM DRILL SIZE      : 7.87 MIL

5) MAXIMUM DRILL SIZE      : 156 MIL

6) MINIMUM TRACK THICKNESS : 4.00 MIL

7) MINIMUM TRACK CLEARANCE : 3.26 MIL

8) COPPER THICKNESS        : AS PER FAB DRAWING

9) FINISH                  : AS PER FAB DRAWING

10)Material                : AS PER FAB DRAWING

11)MINIMUM ANNULAR RING/PAD SIZE: 4.0 MILS/16.00 mils

12) ASPECT RATIO		   : 8.51 MILS

13) VIA ON PAD TECHNOLOGY  : NOT PRESENT

OTHER LAYERS

------------

FAB.GBR		- FAB DRAWING

ROUT.GBR		- OUTLINE

IPC NETLIST.IPC 	- IPC D 356 NETLIST

K2E Stackup_20131011.XLS    - Stackup Details

EINF4984B_K2L_EVM_CARD_odb++.tgz	    - ODB++ DATA

NOTE:

1)THIS IS A CONTROLLED IMPEDANCE BOARD.FOR DETAILS PLEASE REFER FAB DRAWING.

2)MASK PADS HAS BEEN INCREASED BY 3MIL WITH RESPECT TO DATABASE AT CAM LEVEL AS PER REQUEST.

3) AGND,GND,GND_3 nets are shorted intentionally in Top side.

4)6 DUPLICATE DRILL PRESENT IN THE BOARD FOR U22.

5)BGA VIAS TENTED CAM LEVEL FOR U80,NAND1, CN16, CN17 AND GPS1.

For any queries on this design please contact

Design Engineer : Raghavendra H

Email id : raghavendra.h@siennaecad.com

Mobile NO :

Tel No : 30410727

CAM Engineer : Madhu.BR/Karthik.R

Email id : madhu.br@siennaecad.com/karthik.r@siennaecad.com

Mobile No:

Tel N0: 30410766