Commands

NOTE: All commands are case insensitive.

- wait n

  - Wait n milliseconds.

  - UART output: The command will print “Delaying n milliseconds…” and will print “ Finished” upon completion.

  - Example:

    - wait 250

    - Output: Delaying 250 milliseconds… Finished

- gpio port [value]

  - Read or write to the specified port.

  - Without value the specified port is read and the value is printed in binary (with '_' every 4 bits for clarity).

  - With value the specified value is written to the port.

  - port can be A, B, C, D, E, F, or G for the MCU onboard GPIO ports or XA, XB, XC, XD for the GPIO expanders. For K2L XA, XB, XC, XD and XE are the SPI to GPIO expanders in FPGA. For more information on these registers and bit to pin mapping info please read K2LEVM_FPGA_Register_Bank.xls provided in the same doc folder.

  - Onboard ports (A,B,C,D,E,F,G) are 8 bits while expander ports(XA,XB,XC,XD,XE) are 16 bit.

  - value with be 8 or 16 bit accordingly.

  - Each digit can be one of the following:

    - 0: output a 0 to this bit

    - 1: output a 1 to this bit

    - z: configure this bit for input

    - x: leave this bit alone

  - All underscores in the input value are ignored.

  - UART output:

    - Write: The command will print “Writing to *port*…” where port is either GPIO expander X or port X. The command will then print the values of the input mask, the high output mask, and the low output mask in hexadecimal format (with leading 0x). Upon completion the command will print “Finished setting *port*”

      - Example: gpio xc xxxx_zzzz_11xx_0000

        - Writing to GPIO expander C…

        - Inputs: 0x0F00

        - High Outputs: 0x00C0

        - Low Outputs: 0x000F

        - Finished setting GPIO expander C

    - Read: The command will print “Reading from *port*…” where port is either GPIO expander X or port X. The command will then print the input values and output values in binary format.

      - Example: gpio a

        - Reading from port A…

        - Inputs: XXXX_XXXX

        - Outputs: XXXX_1XXX

  - Examples:

    - gpio a 1xx0_zz00

    - gpio B

    - gpio XA 1100_0011_xxxx_zzzz

    - gpio xc

- spi port. cs value

  - Simultaneously read and write value to the specified SPI port.

  - port can be 0 or 1 to indicate which SPI interface to use.

  - cs indicates to chip select that will be used for SPI communication.

  - value should be given in bytes (in hexadecimal format) separated by spaces.

  - UART output: The command will print “Sending data through SPI port CS cs…”. It will then print “Sending value…” before sending each value and “ Sent” upon a successful send of the value. It will then print “Reading results…” followed by printing all the results on one line, in hexadecimal format, separated by spaces.

    - Example: spi 0.5 41 12 00 00 00 00

      - Sending data through SPI0 CS5…

      - Sending 0x41… Sent

      - Sending 0x 12… Sent

      - Sending 0x 00… Sent

      - Sending 0x 00… Sent

      - Sending 0x 00… Sent

      - Sending 0x 00… Sent

      - Reading results…

      - Results: 41 12 80 00 80 00 7F

  - Examples:

    - spi 0.3 41 00 01 02 03

    - spi 1.0 00 00 01 00

- i2c port. address ( value or r)

  - Performs read and write operations on the specified I2C port. If no device of the same I2C address is found, it will throw error.

  - If value is given, the specified value is written to the specified device.

  - If r is given in place of value, a read operation is performed on the specified device.

  - port can be 0 or 1 to indicate which I2C interface to use. Realisitically this will always be 1 as I2C0 will operate in slave mode.

  - Address is the slave address of the specified device (in hexadecimal format).

  - UART output:

    - Write: The command will print “Writing to I2C port Slave: address…”. The command will then print “Sending value…” before sending each value. Upon a successful send, the command will print “ Sent.” If the I2C times out, however, the command will print out an error, and exit. After a complete successful send, the command will print “Send command complete.”

      - Example: i2c 1.0 00 00 00

        - Writing to I2C1 Slave: 0x00…

        - Sending 00… Sent

        - Sending 00… Error: I2C Send timed out

        - Exiting I2C command

    - Read: The command will print “Reading from I2C port Slave: address...” and will then begin printing the data received.

      - Example: i2c 1.fa

        - Reading from I2C1 Slave: 0xFA…

        - Data: 6E

  - Examples:

    - i2c 1.F0 r

    - i2c 1.52 41 F3 65

- clkreg clk. reg [ _data_]

  - Reads or writes to the specified clock register.

  - data is a 16 bit value, and can be listed as octal, decimal, or hexadecimal. Using a leading 0 will indicate octal, while using a leading 0x will indicate hex.

  - If data is given, the specified data is written to the specified clock register.

  - If data is not given, the specified clock register is read and the result output on the terminal.

  - clk specifies the clock to be used. Clocks range from 1 - 3.

  - reg specifies the register to be operated on. The register can be listed as octal, decimal, or hexadecimal. Using a leading 0 will indicate octal, while using a leading 0x will indicate hex.

  - UART output:

    - Write: The command will print out “Writing value to register reg on clock module clk…” and append “ Finished” to this line when the write is successful.

      - Example: clk 1.0 0x020F

        - Writing 0x020F to register 0 on clock module 1… Finished

    - Read: The command will print out “Reading from register reg on clock module clk…” and then output the data on the following line.

      - Example: clk 3.5

        - Reading from register 5 on clock module 3…

        - Data: FE56

  - Examples

    - clkreg 1.0 0x020F

    - clkreg 3.5 017

    - clkreg 2.4

- lcd string page line

  - The LCD works through a page system. There are a total of 16 pages (starting at page 0 and ending at page 15), each containing two lines (0 and 1). The command works by writing the given string to the specified page and line.

  - Note: Currently all strings printed through this command are printed in lower case).

  - UART output: Prints the line “Adding “string” to page page line line…” and adds “ Finished” upon completion

  - Example:

    - lcd hello 7 0

    - Output: “Adding “hello” to page 7 line 0… Finished”

- setboot mode

  - Sets the bootmode to the value of mode.

  - Mode is a 32 bit value represented in hexadecimal. Please visit K2E, K2L or K2H HW Setup Guide for information on various bit values.

  - Example:

    - Setboot 10AF1EE1

- bootmode <#bootmode index> <hi_ value> <low_value> <User name of the bootmode>

  - Sets or print the bootmode value.

  - <#bootmode index> is the value ranges from 8 to 15(User Bootmode).

  - <hi_value> is the 32 bit Most significant word, usually set to 0x00000000.

  - <low_value> is the 32 bit Lower significant value of the bootmode that will replace the current bootmode low_value.

  - <User name of the bootmode > is the string that will be identifier of newly replaced bootmode.

  - Example:

    - Bootmode #15 0x00000000 0x08100001 My DSP NoBoot

      - Sets bootmode #15 with respective new high_value, low_value and new name-My DSP NoBoot

    - Bootmode all

      - Prints all the bootmodes stored in the internal LM3s2d93 flash memory.

- warmrst

- fullrst

  - Places the board in reset for 1 second, places the bootmode on the GPIO pins, and then takes the board out of reset.

  - Example:

    - Fullrst

- Reboot [por|pwr|full]

  - Reboot the board, not BMC, according to the option supplied.

- Eeprom addr [bytes…]

  - Writes and reads to the EEPROM

  - If [bytes..] are given as byte-wide values in hexadecimal separated by spaces, then the values are programmed into EEPROM starting at addr

  - If [bytes…] are not given, then the first 16 bytes starting at address are read from eeprom and output on a single line separated by spaces

  - Addr is given as a 2-byte-wide hex value.

  - Example:

    - Eeprom 0F56 A5 F8 70

    - Eeprom 0065

- Commission board_type board_version board_sn

  - Stores the 3 values (given in hexadecimal) into the OTP flash registers, and sets up the BOOTCFG register to allow in-field firmware updates via UART. If these registers have already been programmed (the command has been used before), this command is disabled. Once the command is used, the MCU must be hard reset for the command to take effect.

  - To reset these registers see page 175 of the Stellaris Data Sheet. This process can also be done easily through the LM Flash Programmer in the Other Utilities tab.

  - To perform an in-field update, connect the MCU_BOOTSELECT jumper to ground and reset the MCU. Make sure you are connected to the MCU UART, and use the LM Flash Programmer (available here) to update the firmware, as detailed below.

    - In the ‘Configuration’ tab, in the interface section, select ‘Serial (UART)’ from the drop-down box on the left.

    - Select the appropriate COM Port, and set the ‘Baud Rate’ to 115200.

    - Set ‘Transfer Size’ to 60, and make sure ‘Disable Auto Baud Support’ is unchecked.

    - In the ‘Program’ tab, Browse to the location of the binary file containing the firmware update, and select it.

    - Leave all other options as default, and press the ‘Program’ button.

    - After the programmer is complete, disconnect the jumper from ground and reset the board.

- Error enable

  - Enables and disables error handling in EVM setup.

  - If enable is 1 then error handling is enabled; if it is 0 then error handling is disabled.

  - If error handling is enabled during setup the BMC will stop setup and exit if anytime an error occurs, without completing setup. If error handling is disabled the BMC will output a message to the UART console and continue performing setup.

  - By default error handling is enabled.

  - This command is volatile and resets to enabled on reset. To perform setup without error handling enabled the ‘pwr’ command will need to be used.

- Pwr

  - This command performs EVM setup.

- Ver

  - Displays the BMC version, board type, board version, and board serial number in a formatted way on the UART console.

  - Additionally UCD configuration version and FPGA bit file version(for K2L EVM only).

- fpga_reg

  - Writes into or reads from 8 bit FPGA registers. This command is for K2L EVM only.

  - Example:

    - fpga_reg  22 r

    - fpga_reg 20 01

- serdes

  - Writes into or read into the 3 serdes Switched in K2L –EVM. This command is for K2L EVM only. Three serdes switches have i2c address 67, 63 and 5F.

  - Example: serdes <i2c addr of SW> <reg addr> <data>

    - serdes 67 41 r

    - serdes 63 02 01

- interface_muxsel

  - Sets/resets the bootmode bit mapped to CSISC2_3_MUXSEL and CSISC2_0_MUXSEL. In K2L, PCIe bus 2 and 3 & SGMII port 2 and 3 are multiplexed, and selection of either is controlled by CSISC2_3_MUXSEL pin at boot time. Similarly, JESD 0 and 1 & AIL are multiplexed, and selection of either is controlled by CSISC2_0_MUXSEL pin of SoC at boot time. This command followed by BMC reset will select the interface. This command is for K2L EVM only. The valid options are pcie | sgmii and jesd | ail. Command without any option will throw current interfaces selected.

  - Example: interface_muxsel [pcie|sgmii] [jesd|ail]

    - interface_muxsel ail

    - interface_muxsel pcie

- clk_prof

  - Sets the user defined clock profiles active or inactive.(BMC hardcoded one). 2 Clock gen ICs CDCM6208 are configured with hardcoded values at bootup time. Additionally there are two profile sets available to user in flash memory region. These can be used as the user defined. Two profiles are there. One for CDCM 0 and another for CDCM 1. If set to “on” than if the valid value is there for respective CDCM , it will be filled. Data with 0xFFFF will be ignored and in those case hardcoded values will be taken. If set to off then User profile will not be considered at all. And hardcoded config set is filled in registers.

  - Example: clk_prof [CDCM 0| CDCM 1] [on|off]

    - Clk_prof 0 off

    - Clk_prof 1 on

    - Clk_prof – Show the status of active or inactive

- Profile_reg

  - Set/reads the user defined clock register in a profile set. There are  profile sets each consisting 21 flash memory locations. The profile for the particular CDCM must be entered first followed by the register number to be changed. Last argument is 16-bit hex value. Once filled and if profile set is active, this value will be filled for the particular register and CDCM. Only useful locations needs to be filled with the user defined values. If filled with 0xFFFF then it will not be counted as a user defined value and for that register case, hardcoded value will be filled. If needs to undefined register, enter 0xffff as a reg data. If no other arguments are given then it will display two sets.

  - Example: profile_reg [CDCM 0|CDCM 1][Reg 0 to 20] [User custom value in hex]

    - profile_reg 0 0 0x01B9

    - profile_reg 1 12 0x1234

    - profile_reg – Display profile sets but only valid